# SNS COLLEGE OF TECHNOLOGY

(An Autonomous Institution)

Approved by AICTE, New Delhi, Affiliated to Anna University, Chennai Accredited by NAAC-UGC with ‘A++’ Grade (Cycle III) &

Accredited by NBA (B.E - CSE, EEE, ECE, Mech & B.Tech.IT)

COIMBATORE-641 035, TAMIL NADU

DEPARTMENT OF AIDS

Course Code & Name : 23EET103 & ELECTRIC CIRCUITS AND ELECTRON DEVICES

Course Faculty	: Ms.RANJANI K

Puzzles / In Class Activities

UNIT V– RECTIFIERS AND POWER SUPPLIES

### Q1. (GATE-Style Numerical Puzzle)

A half-wave rectifier feeds a resistive load RL. If the diode has a forward resistance equal to RL, the efficiency of the rectifier is closest to:

• Answer: ≈ 20.7%
• Industry Mapping: Texas Instruments
• GATE Mapping: GATE ECE 2014 (modified)

### Q2. (Concept Trap – Deep Understanding)

Why does a bridge rectifier exhibit lower PIV per diode compared to a center-tapped full-wave rectifier for the same DC output?

• Answer:
Each diode blocks only peak secondary voltage Vm, whereas center-tapped diodes must block 2Vm due to reference to transformer midpoint.
• Industry: Analog Devices
• GATE: GATE ECE 2017

### Q3. (Ripple-Frequency Logic Puzzle)

For a 60 Hz input supply, the dominant ripple frequency at the output of a π-filter following a full-wave rectifier is:

• Answer: 120 Hz
• Industry: Qualcomm
• GATE: GATE ECE 2019

### Q4. (Multi-Concept Numerical)

A full-wave rectifier delivers 1 A DC current. If ripple factor is 0.48, estimate RMS ripple current.

• Answer:Ir=r×Idc=0.48A
• Industry: Intel
• GATE: GATE ECE 2018

### Q5. (Non-Ideal Diode Puzzle)

If diode forward voltage increases, which rectifier suffers maximum reduction in DC output voltage?

• Answer: Half-wave rectifier
• Industry: ON Semiconductor
• GATE: GATE ECE 2015

### Q6. (Filter Design Puzzle)

Why is an inductor preferred over a capacitor for filtering high current rectifier outputs?

• Answer:
Inductor resists change in current and avoids high surge current, improving regulation under heavy load.
• Industry: ABB
• GATE: GATE ECE 2016

### Q7. (Advanced π-Filter Puzzle)

In a π-filter, increasing the second capacitor mainly improves:

• Answer: Load regulation and ripple suppression
• Industry: Murata
• GATE: GATE ECE 2018

### Q8. (Zener Regulator Stress Puzzle)

A Zener regulator operates at no load. Which component experiences maximum power dissipation?

• Answer: Zener diode
• Industry: STMicroelectronics
• GATE: GATE ECE 2017

### Q9. (Dropout Voltage Logic)

Why is dropout voltage a non-issue in ideal switching regulators?

• Answer:
Because regulation depends on duty cycle, not linear voltage drop.
• Industry: Texas Instruments
• GATE: GATE ECE 2020

### Q10. (Thermal-Electrical Coupling)

Increasing ambient temperature affects linear regulators primarily by:

• Answer: Reducing maximum allowable load current
• Industry: Infineon
• GATE: GATE ECE 2019

### Q11. (Efficiency Puzzle – Tough)

Why does efficiency of SMPS remain high even at large input–output voltage differences?

• Answer:
Because power loss depends on switching and conduction losses, not voltage drop.
• Industry: Apple Power Team
• GATE: GATE ECE 2021

### Q12. (Switching Loss Puzzle)

Increasing SMPS switching frequency increases which loss most significantly?

• Answer: Switching loss
• Industry: NVIDIA
• GATE: GATE ECE 2020

### Q13. (Converter Identification Puzzle)

An SMPS produces output voltage greater than input using a single inductor. Identify the topology.

• Answer: Boost converter
• Industry: Qualcomm
• GATE: GATE ECE 2018

### Q14. (Duty-Cycle Trap)

If duty cycle of a buck converter exceeds 0.5, what happens to output voltage?

• Answer: Output increases proportionally (Vout = D·Vin)
• Industry: Broadcom
• GATE: GATE ECE 2017

### Q15. (UPS Transfer Logic)

Why does offline UPS have non-zero transfer time?

• Answer:
Load is switched mechanically/electronically from mains to inverter.
• Industry: Schneider Electric
• GATE: GATE ECE 2016

### Q16. (Battery Charging Curve Puzzle)

Which charging mode dominates during initial charging of lead-acid batteries?

• Answer: Constant current mode
• Industry: Exide Industries
• GATE: GATE ECE 2018

### Q17. (Thermal Resistance Network Puzzle)

Total thermal resistance of series thermal paths is:

• Answer: Sum of individual resistances
• Industry: Intel Thermal Group
• GATE: GATE ECE 2019

### Q18. (Heat Sink Optimization)

Why is black anodizing used in heat sinks?

• Answer:
Improves radiative heat transfer.
• Industry: AMD
• GATE: GATE ECE 2020

### Q19. (Reliability Puzzle)

Which failure mode is accelerated most by poor thermal management?

• Answer: Semiconductor junction breakdown
• Industry: NVIDIA
• GATE: GATE ECE 2021

### Q20. (System-Level Design Challenge)

Why is SMPS preferred over linear supply in aerospace systems?

• Answer:
High efficiency, low weight, compact size.
• Industry: ISRO / Boeing
• GATE: GATE ECE 2022

### Q21. (Rectifier + Filter Interaction)

Why does capacitor-input filter worsen transformer regulation?

• Answer:
High peak charging currents cause voltage drop in transformer winding.
• Industry: Siemens
• GATE: GATE ECE 2018

### Q22. (Ripple Current Stress)

High ripple current in capacitors leads to:

• Answer: Heating and reduced capacitor life
• Industry: Panasonic
• GATE: GATE ECE 2019

### Q23. (EMI Puzzle)

Why does SMPS require EMI filters?

• Answer:
High-frequency switching generates conducted and radiated noise.
• Industry: Bosch
• GATE: GATE ECE 2020

### Q24. (Numerical Trap)

If filter capacitance is doubled, ripple voltage becomes:

• Answer: Half
• Industry: Texas Instruments
• GATE: GATE ECE 2016

### Q25. (Ultimate Integration Puzzle)

A medical device requires: zero ripple, high efficiency, zero interruption, and safe temperature.
Best configuration?

• Answer:
SMPS + LC filter + Online UPS + Active cooling
• Industry: GE Healthcare
• GATE: GATE ECE 2023